# Egypt carbon pricing and CBAM: results and methodology

# 1. Results at a glance

| Results in 2030 | 1A | 2A | 2B | 3A | 3B | 3C |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario | Comprehensive levy | Upstream levy | Upstream levy, household refunds | Downstream industry price | 3A with feebate (revenue neutral) | 3A with abatement fund |
| What is priced | Energy CO2, all sectors, plus CBAM process emissions | Energy CO2, all sectors | Energy CO2, all sectors | CBAM products: energy and process emissions | As 3A | As 3A |
| Use of revenue | Public investment | Public investment | Household refunds | Household refunds | Returned to firms per unit of output | Abatement fund for industry |
| Carbon price ($/t CO2) | 20 | 20 | 20 | 20 | 20 | 20 |
| Share of national emissions priced1 | 71.3% | 64.2% | 64.2% | 13.5% | 13.5% | 13.5% |
| Carbon revenue, $bn | 5.6 | 5.0 | 5.0 | 1.0 | 0 | 0 |
| Emission cut, Mt CO2e | 39.8 | 36.8 | 40.4 | 6.9 | 5.1 | 11.5 |
| Cut as share of national emissions | 9.1% | 8.5% | 9.3% | 1.6% | 1.2% | 2.6% |
| Share of heavy-industry emissions priced | 100.0% | 44.2% | 44.2% | 100.0% | 100.0% | 100.0% |
| Heavy industry: total emissions | −11.9% | −6.8% | −6.8% | −11.9% | −8.7% | −19.6% |
| of which emissions intensity | −8.7% | −5.2% | −5.2% | −8.7% | −8.7% | −16.6% |
| of which output, emissions-weighted | −3.5% | −1.7% | −1.7% | −3.5% | 0.0% | −3.5% |
| EU border charge per tonne exported | −38.7% | −20.7% | −20.7% | −38.7% | −8.5% | −43.9% |
| of which emissions intensity of EU exports4 | −8.5% | −5.7% | −5.7% | −8.5% | −8.5% | −16.2% |
| of which Egyptian carbon price credit | −30.2% | −15.0% | −15.0% | −30.2% | 0.0% | −27.6% |
| Premature deaths avoided in 2030 | 1,617 | 1,597 | 1,757 | 52 | 38 | 86 |

## How the results are worked out

The World Bank’s Climate Policy Assessment Tool (CPAT) simulates each policy for the whole economy. It treats the CBAM products only roughly, so they are modeled separately (sections 5 and 6), and the EU border charge is computed per tonne exported (section 7). A carbon price affects each CBAM product in two ways:

- Output falls a little. The price elasticity of physical output is the percentage fall in output per 1% rise in price: −0.1 for cement, which is barely traded; −0.4 for steel and fertilizers; and −0.5 for aluminum, which is priced on world markets (Appendix C).

- Each tonne gets cleaner. Fuel emissions per tonne, for heat and as a reductant, fall about 0.58% per $1 of carbon price: CPAT’s industry fuel response less the block’s own output effect (Appendix B). Process emissions per tonne fall 0 to 0.8% per $1, from IPCC abatement cost data (Appendix C).

Example: a downstream price of $20/t CO2 on fuel and process emissions

|  | DRI-EAF steel | Cement (clinker) |
| --- | --- | --- |
| Emissions intensity before the price, fuel + process (tCO2e/t) | 0.571 + 0.038 | 0.314 + 0.537 |
| Carbon cost per tonne (share of price) | $12 (2.3%) | $17 (48.6%) |
| Output | −0.9% (εi=−0.4) | −3.9% (εi=−0.1) |
| Fuel emissions intensity | −11.0% (βf=0.0058) | −11.0% (βf=0.0058) |
| Process emissions intensity | −6.6% (βinp=0.0034) | −6.4% (βinp=0.0033) |
| Total emissions intensity | −10.7% | −8.1% |
| Total emissions | −11.5% | −11.7% |

Most of the cut comes from cleaner production; output matters only for cement, where carbon is about half the clinker price. Levels for 3A to 3C can move by about 30% with the output data (Appendix E).

## What the results say

- A broad price does most. Pricing fuels across the economy cuts national emissions by about 9%, raises about $5 billion a year and avoids 1,600 to 1,760 early deaths. A price on heavy industry alone cuts 1 to 3% and raises about $1 billion.

- Pricing process emissions adds to the cut. Pricing them too (1A against 2A) adds about 7 points of coverage and 3.0 Mt.

- Heavy industry gets cleaner. At $20, its emissions intensity falls about 9%; an abatement fund (3C) roughly doubles that.

- Refunding firms keeps most of the abatement. In 3B output is protected and revenue is zero, but intensity falls as in 3A, so the cut is about a quarter smaller (5.1 against 6.9 Mt).

- Crediting Egypt’s price matters most for the border charge. Cleaner production trims the charge per tonne by about 9%; with Egypt’s carbon price credited, the fall is 38.7% in 1A. An output-based rebate (3B) earns no credit; an abatement fund (3C) neither adds to it nor cancels it.

# 2. Purpose and approach

Each result is the CPAT answer plus a short, explicit list of adjustments for the seven CBAM products, which CPAT treats only roughly. All dollar figures are US dollars.

The study reports nine results for 2030 in each of six scenarios, written in sections 6 and 7 as: coverage of national GHG (C); carbon revenue (R); the change in national GHG (ΔEtot, so the emission cut is −ΔEtot) and its share of the baseline (ΔEtot/Etot0); the change in CBAM block emissions (ΔECBAM/ECBAM0), in their emissions intensity (I) and their priced share (S); the change in the CBAM obligation per tonne exported (ΔΩ/Ω0); and premature deaths avoided (D).

CPAT models the energy and health side of every scenario. It has no output, trade or process-emission response, so the CBAM block is calculated outside CPAT, in three steps.

- Read the scenario’s CPAT results for 2030.

- Calculate how the seven CBAM products respond to the carbon price: output, fuel emissions and process emissions, including any fund.

- Walk from the CPAT result to each final result in a few labeled steps: remove CPAT’s own estimate for the CBAM block, then add the block calculation.

Some terms and codes are used throughout, so they are introduced here.

Terms. CPAT is the World Bank’s Climate Policy Assessment Tool. The CBAM block is the seven products named in section 3, which fall under the EU’s Carbon Border Adjustment Mechanism (CBAM). DRI-EAF steel is steel made from direct reduced iron in an electric arc furnace, scrap-EAF steel is made from scrap in the same furnace, and BF-BOF is blast furnace steel, which is not modeled. IPPU is CPAT’s industrial processes and product use category, which holds process emissions. CPAT Team (2024) is CPAT’s technical documentation.

Emission factor codes. Each product has four emission factors, in tonnes of CO2-equivalent per tonne of product: fuel burned for heat (code fc, σfc); fuel carbon used as a reductant or feedstock, such as the reducing gas in DRI or the gas fed to an ammonia reformer (fp, σfp); non-fuel process CO2, mainly from limestone, electrodes and anodes (np, σnp); and non-CO2 process gases, N2O from nitric acid and PFCs from aluminum smelting (no, σno). Fuel emissions per tonne are σf=σfc+σfp and process emissions per tonne σp=σnp+σno. Appendix D explains how they are derived.

Scenario settings. The carbon price τ is $20 per tonne of CO2 in 2030. The process switch π is 1 when process emissions are priced. The rebate share θ is 1 when the cost is refunded in full per unit of output. The fund switch φ is 1 when a fund adds a shadow price ψ to efficiency decisions. Section 3 sets these for each scenario.

The workbook follows the same structure. Its Calculations sheet opens with the walks, followed by the inputs and supporting numbers; the Product detail sheet holds the seven-product calculation; and the Results sheet repeats the final line of each walk.

# 3. Scenarios and core assumptions

Six scenarios are run. Each reads one CPAT run and sets a few switches in the block calculation.

| Scenario | CPAT run | What is priced | Process emissions priced | Use of revenue |
| --- | --- | --- | --- | --- |
| 1A Comprehensive levy | EGY1A | Energy CO2 in all sectors, plus CBAM process emissions | Yes | Public investment |
| 2A Upstream levy | EGY2A | Energy CO2 in all sectors | No | Public investment |
| 2B Upstream levy | EGY2B | Energy CO2 in all sectors | No | Household refunds |
| 3A Downstream industry price | EGY3A | Energy CO2 and process emissions of the CBAM products | Yes | Household refunds |
| 3B 3A with feebate | EGY3B | As 3A, with an output-based rebate (θ=1) | Yes | Returned to firms per unit of output |
| 3C 3A with abatement fund | EGY3C | As 3A, with a fund (φ=1) | Yes | Abatement fund for industry |

The price is $20 per tonne of CO2 in 2030 in every scenario. In 3A to 3C, diesel and gasoline are excluded from the price; other oil products are priced.

The core assumptions are these.

- The block covers seven products: DRI-EAF steel, scrap-EAF steel, gray clinker, ammonia, urea, ammonium nitrate and primary aluminum. BF-BOF steel is a reference only and has no output.

- A priced scenario passes the carbon cost into the product price in full. A rebate returns a share θ of the cost per unit of output, so only the net cost reaches the output decision. The intensity response still sees the full price.

- Output follows a constant-elasticity demand curve, with elasticities by product (Appendix C).

- Fuel and process emissions per tonne fall with the carbon price along fixed semi-elastic curves (Appendices B and C).

- The fund in 3C works as a shadow price on efficiency, added to the carbon price in the intensity responses only, as in CPAT (CPAT Team 2024, section 3.1.8.1). Its value is fixed at $20 per tonne.

- Product prices, 2024 output, growth and EU exports are exogenous and identical in every scenario.

- Emission factors per tonne are fixed technology values that change only through the responses above.

- Under the CBAM rules, an explicit carbon price can be credited against the obligation; an output-based rebate (3B) makes it ineligible; a decarbonization fund (3C) neither adds to the price credited nor invalidates it.

- The CBAM obligation never changes the emission results.

# 4. Notation

Table 4.1 lists the notation, grouped by kind; each symbol is also defined where it first appears. Emissions intensity is written σ, as in Nordhaus’s DICE model.

Table 4.1 Notation

| Symbol | Meaning |  | Symbol | Meaning |
| --- | --- | --- | --- | --- |
|  | Conventions |  |  | Emissions intensity (tCO2e/t) |
| i | One of the seven CBAM products |  | σifc, σifp, σif | Fuel: heat; reductant; total |
| xtot, xCBAM | Scope: national total, energy (en), industry (ind), CBAM block |  | σinp, σino, σip | Process: CO2; non-CO2; total |
| x0, Δx | Baseline value; change from baseline |  | σi | Total emissions intensity |
| x | CPAT’s own value, before adjustment |  | σ, r | EU-export average; its fall |
|  | Policy settings |  |  | Emissions (Mt CO2e) |
| τ | Carbon price ($20/t in 2030) |  | Eif, Eip | Fuel and process, by product |
| π | 1 if process emissions are priced |  | ECBAMf, ECBAMp | CBAM block: fuel, process |
| θ | Rebate share (1 in 3B) |  | ECBAM, ECBAM | Block total; at baseline intensity |
| φ | 1 if revenue goes to a fund (3C) |  | Etot | National GHG |
| ψ | Fund shadow price ($20/t) |  | Een, Eind | National, industry energy CO2 |
|  | Prices and carbon paid ($/t) |  | af | Fuel adjustment, ΔECBAMf−ΔECBAMf |
| pi | Product price before the policy |  | ω | CPAT priced share, energy CO2 |
| δi | Net carbon cost as a share of pi |  |  | Revenue, health and obligations |
| di, d | Carbon paid per tonne; export average |  | R, RB | CPAT receipts; block payments ($m) |
| pEU, γ | EU certificate price; phase-in factor |  | D | Deaths avoided (CPAT) |
| τEU, τEGY | EU price after phase-in; Egyptian price credited |  | Ω | Obligation per tonne exported ($/t) |
| κ | Deduction scaling (1 = full credit) |  |  | Results |
|  | Responses |  | C | Coverage of national GHG |
| εi | Price elasticity of physical output |  | R | Carbon revenue ($m; $bn in section 1) |
| βf | Fuel-intensity semi-elasticity (0.0058) |  | −ΔEtot | Emission cut (Mt) |
| βinp, βino | Process semi-elasticities (0 to 0.0079) |  | ΔEtot/Etot0 | National GHG change |
|  | Output and trade |  | ΔECBAM/ECBAM0 | Block emissions change |
| Yi, Yi2024 | Output in 2030 and in 2024 (Mt) |  | I | Block intensity change |
| gi | Output growth (% a year) |  | S | Priced share, block emissions |
| Xi | EU exports by product (Mt) |  | ΔΩ/Ω0 | Obligation change per tonne |
|  |  |  | D | Premature deaths avoided in 2030 |

# 5. The CBAM block model

The block is calculated product by product in six steps, then added up. Output and emissions are in Mt and prices in $/t, so emissions are output times intensity, and carbon payments come out in $ million.

Step 1. Net cost increase, as a share of the price, after any rebate (δi is the net carbon cost for product i as a share of its price pi; θ the rebate share; τ the carbon price; σif and σip fuel and process emissions per tonne; π the process switch):

δi=1−θ τ σif+πσippi

Step 2. Output. Baseline output grows from 2024 for six years, and the net cost moves it along a constant-elasticity curve (Yi is output; gi its growth rate; εi the price elasticity of physical output; superscript 0 the baseline):

Yi0=Yi20241+gi6,  Yi=Yi0 1+δiεi

Step 3. Fuel emissions fall with the full price plus any fund shadow price (Eif is fuel emissions in Mt; σifc and σifp fuel burned for heat and fuel used as reductant or feedstock, per tonne; βf the fuel-intensity semi-elasticity; φ the fund switch; ψ the fund shadow price):

Eif=Yi σifc+σifp e−βfτ+φψ

Step 4. Process emissions fall if they are priced, or with the fund (Eip is process emissions; σinp and σino process CO2 and non-CO2 per tonne; βinp and βino their semi-elasticities):

Eip=Yiσinpe−βinpπτ+φψ+σinoe−βinoπτ+φψ

Step 5. Emissions intensity (σi), emissions per tonne:

σi=Eif+EipYi

Step 6. Carbon paid per tonne, net of any refund, the basis for a CBAM credit (di):

di=τ Eif+πEipYi−θ τ σif+πσip

Output responds to the net price, fuel intensity to the full price and the fund, and process intensity to the process price and the fund, so nothing is counted twice. The products are then added up into block totals, ECBAMf=i​Eif and ECBAMp=i​Eip, and export-weighted averages.

# 6. The walks: from CPAT to each result

Each result starts from CPAT and makes a few signed adjustments. Emissions are written E, with a subscript for scope and a superscript for fuel (f) or process (p); a hat marks CPAT’s own value (Table 4.1). The workbook’s Calculations sheet shows each walk line by line.

CPAT’s own change for the CBAM products (ΔECBAMf, ΔECBAMp) is its change in industry energy CO2 and IPPU in a run pricing only those sectors: each run’s own in 3A to 3C, and 3A’s in 1A to 2B.

Walk 1. Emission change, ΔEtot. CPAT’s change in national GHG, ΔEtot, less its own change for the CBAM products, plus the block’s own changes. The emission cut in section 1 is −ΔEtot:

ΔEtot=ΔEtot−ΔECBAMf+ΔECBAMp+ΔECBAMf+ΔECBAMp

Walk 2. Share of national GHG. The change as a share of baseline national GHG is ΔEtot/Etot0. It uses the 2022 inventory’s energy share: Etot0=Een0/0.65 = 435 Mt (CPAT: 536 Mt).

Walk 3. Carbon revenue, R. CPAT’s receipts, corrected for the CBAM products, plus process fees, less refunds and fund outlays (R is CPAT’s carbon receipts; af=ΔECBAMf−ΔECBAMf the fuel adjustment; RB the block’s carbon payments):

R=R+τ af1A to 2BτECBAMf−R3A to 3C+πτECBAMp−θ+φ RB

Walk 4. Deaths avoided, D (in 2030, not cumulative). CPAT’s count D is scaled by the fuel adjustment relative to its national energy CO2 change ΔEen. In 3A and 3C, where CPAT’s counts are distorted by fuel switching, 3B’s deaths are scaled by the emission change (superscript 3B marks the 3B value):

D=D1+afΔEen for 1A to 2B and 3B;  D=D3B ΔEtotΔEtot3B for 3A and 3C

Walk 5. Coverage of national GHG, C. In 1A to 2B, CPAT’s priced share ω of baseline energy CO2, plus block process emissions when priced. In 3A to 3C, which price only the CBAM products, the block’s own baseline emissions (Appendix G):

C=ω Een0+πECBAMp,0Etot0 for 1A to 2B;  C=ECBAM0Etot0 for 3A to 3C

Walk 6. CBAM block emissions. ECBAM=ECBAMf+ECBAMp is block emissions, and ECBAM the same at baseline intensity and policy output. The results are the change in block emissions, ΔECBAM/ECBAM0; the change in emissions intensity, I; and the priced share of block emissions, S:

ΔECBAMECBAM0=ECBAMECBAM0−1,  I=ECBAMECBAM−1,  S=ECBAMf+πECBAMpECBAM

# 7. CBAM obligations

The obligation is measured per tonne of EU exports, so export volume does not enter. Three rules decide whether Egypt’s carbon price can be credited against it: an explicit carbon price counts; an output-based rebate (the 3B feebate) makes it ineligible; and an abatement fund (3C) neither adds to the price credited nor invalidates it.

Walk 7. Obligation per tonne exported, Ω. The EU price after phase-in, times the emissions intensity of EU exports, minus the Egyptian carbon price where it is eligible (γ is the 2030 CBAM phase-in factor, 0.485; pEU the EU certificate price, $125/t; Xi EU exports of product i; σi its emissions intensity; σ the export-weighted average; κ the deduction scaling, 1 for full credit; d export-weighted carbon paid per tonne; 1−θ the eligibility, zero in 3B). The result is the change ΔΩ/Ω0:

σ=i​Xi σii​Xi,  Ω=max0, γ pEU σ−1−θ κ max0,d,  ΔΩΩ0=ΩΩ0−1

The change splits into a cleaner-exports part, σ/σ0−1, and a carbon price credit. The credit is the carbon actually paid per tonne, so it shrinks as exports get cleaner; this is why 3C’s credit is a little smaller than 1A’s. The fund shadow price ψ is not part of d, so the fund adds nothing to the credit.

Walk 8. A simple check. Cleaner exports cut the obligation by r; the Egyptian price is then credited against the EU price on the priced share S (r=1−σ/σ0 is the fall in σ; τEGY the Egyptian carbon price credited; τEU the EU carbon price after phase-in):

ΔΩΩ0≈− r−1−r τEGYτEU,  τEGY=τS1−θ, τEU=γ pEU≈$61/t

The ratio is capped at 100%. The check matches Walk 7 where the whole block is priced (1A, 3A, 3C). In 2A and 2B it credits the priced share of the price rather than the carbon actually paid per tonne exported, so it differs slightly.

# 8. What drives the results, and limitations

Five things move the results, in rough order of influence.

- Which sectors are priced. The 3A to 3C runs price only the CBAM sectors, so coverage of national GHG is about 14% and the cut is 5 to 12 Mt, against 64 to 71% coverage and 37 to 40 Mt in 1A to 2B. This is the right scale for a heavy-industry policy; v1.0’s larger base came from CPAT pricing all industrial fuel (Appendix G).

- Whether process emissions are priced. Process emissions of cement, steel, aluminum and fertilizer are a large part of block emissions. Pricing them is why 1A covers 71% of national GHG against 64% in 2A and cuts about 40 Mt against about 37 Mt.

- The fuel-intensity and process responses. The fuel-intensity semi-elasticity βf (Appendix B) and the process semi-elasticities (Appendix C.2) set how much emissions per tonne fall. They drive the change in emissions intensity and the border charge, and most of the change in block emissions. The fund in 3C roughly doubles the fall in emissions per tonne (about −17% against −9%).

- The carbon price relative to the product price. At $20 per tonne, carbon is about 2% of the price of steel and aluminum and 1 to 10% of fertilizer prices, so their output barely moves. Clinker is the exception: carbon is about half its price, so cement’s price elasticity of physical output is the output parameter that matters most.

- Revenue recycling and the credit rule. In 3B the feebate refunds the cost, so net revenue and the output effect are zero by construction while intensity still responds to the full price. The border charge per tonne falls only about 9% from cleaner exports, but about 39% in 1A once the Egyptian carbon price is credited, so the credit rule matters far more than the abatement response.

The results have these limits.

- CPAT’s long-run elasticities take full effect within one year, and industrial output is inferred rather than modeled, so output effects are an upper bound.

- There is no trade or leakage model. CBAM exposure is computed outside CPAT.

- Process responses come from global IPCC cost data, not Egyptian abatement cost curves, and one fuel parameter stands for all fuels and products. That parameter is a residual: CPAT’s response for all industry less the block’s output effect, so it assumes the CBAM products respond like average industry. Appendix F compares it with the literature.

- Emission factors are fleet averages, not plant distributions. Output and EU exports rest on published sources and growth on IMF forecasts and announced capacity (Appendix A), prices are 2024 market benchmarks held constant, and the product splits within steel and fertilizers are assumptions, to be replaced with CAPMAS (Egypt’s statistics agency) and UN Comtrade data.

- In 3A to 3C, diesel and gasoline are excluded from the carbon price. Coverage is the block’s own emissions, and deaths avoided in 3A and 3C scale those of 3B by the emission cut, because CPAT cannot isolate the CBAM products and its runs give implausible negative deaths for these scenarios (see the note to the table in section 1).

- In 3C the fund is simplified: ψ is fixed at $20, so the intensity effect is smaller than a fully solved fund would give.

- Revenue is gross carbon revenue, before effects on existing taxes and subsidies.

# Appendix A. Parameters and sources

Every number below is an input cell in the workbook.

| Parameter | Value | Source | Status |
| --- | --- | --- | --- |
| Carbon price τ | $20/t in 2030 | Study design | Policy choice |
| Emission factors | See Appendix D | Egyptian technology averages; IPCC 2006 Tier 1; CBAM rules | Medium to low |
| Fuel-intensity semi-elasticity βf | 0.0058 per $/t | CPAT’s 1A industry fuel response (0.0068) less the block’s output effect (0.0010); see Appendix B | Derived |
| Price elasticity of physical output εi | −0.1 to −0.5 | Study judgment; see C.1 | Low to medium |
| Process semi-elasticities βinp, βino | 0 to 0.0079 per $/t | IPCC AR6 and AR4 WGIII; see C.2 | Medium to low |
| Fund shadow price ψ | $20/t | Study assumption; CPAT Team (2024) | Simplified |
| 2024 output | DRI-EAF 6.4 Mt; scrap-EAF 4.3 Mt; clinker 40 Mt; fertilizers 6.2 Mt; aluminum 0.3 Mt | Midrex (DRI); worldsteel (steel); USGS (cement; clinker ratio 0.8); EBRD (fertilizers); Egyptalum (aluminum); splits assumed | Sourced |
| EU exports | Steel 0.93 Mt; clinker 0.8 Mt; fertilizers 1.45 Mt; aluminum 167 kt | UN Comtrade (steel); EBRD (fertilizers); Egyptalum (aluminum); clinker assumed | Mostly sourced |
| Growth, 2024 to 2030 | Scrap-EAF and clinker 5%; fertilizers 5.8%; DRI-EAF 2%; aluminum 0% a year | IMF GDP forecasts (about 5%) where capacity allows; announced fertilizer capacity (+2.5 Mt); DRI at capacity plus half of the idle ESISCO plant; no new aluminum line | Sourced, with judgment |
| Prices (2024) | Steel $530/t; clinker $35/t; ammonia $400/t; urea $340/t; ammonium nitrate $320/t; aluminum $2,600/t | SteelOrbis (Egyptian billet); CW Research (Mediterranean clinker FOB); Argus and QCIntel (Middle East ammonia); World Bank Pink Sheet (urea); Egypt’s ammonium nitrate to urea price ratio; Egyptalum export value | Sourced; ammonia approximate |
| CBAM phase-in factor γ | 0.485 in 2030 | Regulation (EU) 2023/956 | Fixed by law |
| EU certificate price pEU | $125/t in 2030 | €109/t, analysts’ average (Reuters, July 2026), at $1.15/€ | Market forecast |
| Deduction scaling κ | 1 (full credit) | Study assumption | Assumption |
| Energy CO2 share of national GHG | 65% | Egypt BTR1 (2024): 2022 GHG 386 Mt, energy 66% | Sourced |

# Appendix B. The fuel-intensity semi-elasticity βf

B.1 The idea. CPAT shows how much industrial fuel CO2 falls with the carbon price, but the fall mixes two things: less output and lower emissions per tonne. The block already calculates the output part with its own price elasticities of physical output (Appendix C.1). What is left is the fall in fuel emissions per tonne, from efficiency and fuel switching, which is what CBAM measures.

B.2 The value. In the 1A run, CPAT’s industry energy CO2 falls from 88.4 to 77.2 Mt at $20. With a price on fuel only (2A), the block’s output effect lowers its fuel emissions from 27.75 to 27.22 Mt (ECBAMf, fuel emissions at baseline intensity and policy output). βf is the difference between the two log changes, per dollar:

βf=−lnEind/Eind0−lnECBAMf,2A/ECBAMf,0τ

βf=ln88.4/77.2−ln27.75/27.2220=0.136−0.01920=0.0058 per $/t

A $20 price cuts fuel emissions per tonne by about 11%. About 85% of CPAT’s industry response is intensity and 15% output, because the products’ output responds little at $20 (Appendix C.1). The workbook calculates this live from the CPAT results. There is no circularity: the block’s output depends on the carbon cost at baseline intensity, not on βf.

B.3 Why the 2A output effect. CPAT’s industry response is a response to fuel prices. In 1A, the charge on process emissions lowers output further, but that is not part of CPAT’s fuel response. Subtracting the 1A output effect instead would give 0.0051.

B.4 Caveats and a cross-check. CPAT’s response is for all industry (88 Mt of energy CO2), of which the block is about a third, so the residual assumes the CBAM products respond like average industry, whose fuel is also mostly natural gas. In the 3B run, where the refund protects activity, CPAT’s industry energy CO2 falls 1.2% against 2.9% in 3A, which suggests a smaller intensity response. The comparison is rough: CPAT’s refund works through energy prices, and both runs exclude diesel and gasoline.

# Appendix C. Output and process responses

C.1 Price elasticities of physical output εi combine market demand, cost pass-through and trade exposure.

| Product | εi | Basis |
| --- | --- | --- |
| Gray clinker | −0.10 | Inelastic demand, low pass-through, little trade. Carbon is about half the price |
| Steel | −0.40 | Moderate demand response and import competition. Carbon is about 2% of the price |
| Fertilizers | −0.40 | Exporters are price takers. Carbon is 1 to 10% of the price |
| Aluminum | −0.50 | World-priced, mostly exported. Carbon is about 2% of the price |

C.2 Process semi-elasticities. Process emissions per tonne follow σipτ=σip0e−βiτ, separately for np and no (βi stands for βinp or βino). Each is fitted to the share of process emissions that known abatement options remove at about $120/t (2024 prices), from IPCC AR6 WGIII (Tables 11.3 and 12.3) and, for N2O and PFCs, IPCC AR4 WGIII Chapter 7.

| Product | Cut at about $120/t | βi (per $/t) | Main options |
| --- | --- | --- | --- |
| DRI-EAF steel | 34% | 0.0034 | Electrode and charge carbon |
| Scrap-EAF steel | 18% | 0.0016 | Electrode and charge carbon |
| Gray clinker | 34% | 0.0033 | Clinker substitution |
| Ammonia, urea | none | 0 | Fuel response only |
| Ammonium nitrate | 62% | 0.0079 | N2O catalysts in nitric acid |
| Primary aluminum | 22% | 0.0020 | Anode and PFC control |

At $20, these cut process intensity by about 2 to 8%, and more for ammonium nitrate.

# Appendix D. Emission factors

D.1 Boundary and categories. Each factor is the direct (Scope 1) emissions of the installation that makes the good, per tonne, on CBAM boundaries: crude steel at the melt shop, clinker at the kiln, the ammonia complex, the urea plant, the ammonium nitrate plant and the primary aluminum smelter. Electricity (Scope 2) is excluded. The four categories are additive, σi=σifc+σifp+σinp+σino; the split assigns carbon to categories and never changes the total.

D.2 Rules.

- Fuel for heat or as an input. Fuel carbon charged into the process as a reductant or feedstock (ammonia reformer feed gas, DRI reducing gas) is σfp; fuel burned for heat is σfc. Carbon retained in DRI is carried into the electric arc furnace and counted there.

- Process CO2 (σnp) is computed from chemistry: 0.785 t CO2 per tonne of CaO and 1.092 per tonne of MgO, with a kiln-dust correction. Electrode, charge-carbon and anode carbon use CBAM carbon contents (1 t carbon = 3.664 t CO2).

- Non-CO2 gases (σno) are kg of gas per tonne times the global warming potentials used by CBAM (IPCC AR5): N2O 265, CF4 6,630, C2F6 11,100.

- Natural gas emits 0.0561 t CO2 per GJ.

- CBAM conventions. CO2 bound in urea is counted at the ammonia plant, so urea’s process CO2 is zero. The N2O from the nitric acid used in ammonium nitrate (0.79 t per tonne of ammonium nitrate) is included in ammonium nitrate’s own non-CO2 factor.

D.3 Derivation by product (t CO2e per tonne).

| Product | Derivation |
| --- | --- |
| DRI (input to steel) | σfp: 10.5 GJ gas × 75% used as reductant × 0.0561, less carbon retained in DRI = 0.372. σfc: 10.5 GJ × 25% burned × 0.0561 = 0.147 |
| DRI-EAF steel | 0.98 t DRI per tonne of steel, plus the furnace: σfc adds 0.6 GJ gas × 0.0561; σfp adds DRI carbon less carbon in the steel; σnp is electrodes, charge carbon and limestone |
| Scrap-EAF steel | Furnace terms only: gas for heat (σfc) and electrodes, charge carbon and limestone (σnp) |
| Gray clinker | σfc: 3.5 GJ/t × fuel-mix factor 0.0896 (coal 45%, petcoke 40%, gas 5%, heavy fuel oil 2%, alternative fuels 8%) = 0.314. σnp: (0.65 CaO × 0.785 + 0.015 MgO × 1.092) × 1.02 kiln dust = 0.537 |
| Ammonia | σfp: 22.5 GJ feed gas × 0.0561 = 1.262. σfc: (35.2 − 22.5) GJ fuel gas × 0.0561 = 0.712 |
| Urea | σfc: about 2 GJ gas × 0.0561 = 0.112; bound CO2 counted at ammonia |
| Ammonium nitrate | σfc as urea, 0.112. σno: 0.79 t nitric acid × 1.259 t CO2e per tonne = 0.994, where nitric acid N2O is (50% × 2.5 + 50% × 7.0) kg × 265, in tonnes, assuming half the plants abate |
| Primary aluminum | σnp: anode 0.44 t × (1 − 0.024 ash) × 3.664 + baking 0.05 = 1.623. σno: (0.10 kg CF4 × 6,630 + 0.01 kg C2F6 × 11,100), in tonnes, = 0.774. σfc: 2.2 GJ × 0.0561 = 0.123 |

D.4 Results.

| Good | fc | fp | np | no | Total |
| --- | --- | --- | --- | --- | --- |
| DRI-EAF steel | 0.178 | 0.393 | 0.038 | 0 | 0.609 |
| Scrap-EAF steel | 0.045 | 0 | 0.044 | 0 | 0.089 |
| Gray clinker | 0.314 | 0 | 0.537 | 0 | 0.851 |
| Ammonia | 0.712 | 1.262 | 0 | 0 | 1.975 |
| Urea | 0.112 | 0 | 0 | 0 | 0.112 |
| Ammonium nitrate | 0.112 | 0 | 0 | 0.994 | 1.107 |
| Primary aluminum | 0.123 | 0 | 1.623 | 0.774 | 2.521 |

D.5 Data quality. Inputs are graded A (plant data, none yet), B (Egyptian statistics), C (technology defaults calibrated to Egypt’s plants) or D (IPCC or international defaults); most are C or D. The factors pass IPCC Tier 1 checks (ammonia 1.69 to 2.10; clinker process CO2 0.50 to 0.55; aluminum process CO2 1.45 to 1.75). Plant data would help most for ammonia gas use, the share of nitric acid N2O abated at Abu Qir, Egyptalum’s PFC rates, the kiln fuel mix and DRI gas use.

# Appendix E. Sensitivity to output levels and growth

The 2024 output levels and growth rates are sourced but uncertain (Appendix A). To see how much they matter, the workbook was re-run four times: growth rates set to zero and doubled, and 2024 output levels for all seven products cut by 30% and raised by 30%. Nothing else changed.

The percentage results do not move. Emissions per tonne, block emissions and the border charge are the same to within 0.1 percentage point in every run, because they depend on output only through the weights between products. The level results move, because the size of the block sets how much of the CPAT answer is replaced. The table shows the emission cut in Mt.

| Test | 1A | 2A | 2B | 3A | 3B | 3C |
| --- | --- | --- | --- | --- | --- | --- |
| Base case | 39.8 | 36.8 | 40.4 | 6.9 | 5.1 | 11.5 |
| Growth rates zero | 38.1 | 35.8 | 39.4 | 5.2 | 3.9 | 8.7 |
| Growth rates doubled | 42.0 | 38.0 | 41.6 | 9.1 | 6.7 | 15.1 |
| 2024 output levels −30% | 37.7 | 35.6 | 39.2 | 4.8 | 3.6 | 8.1 |
| 2024 output levels +30% | 41.9 | 38.0 | 41.6 | 9.0 | 6.7 | 15.0 |

Two patterns stand out. First, the CBAM-sector-only runs (3A to 3C) are the most sensitive, because the block is most of what is priced: 3A’s cut ranges from 4.8 to 9.1 Mt, about ±30% around the base. Second, 1A to 2B move by 3 to 6%, because CPAT carries most of the answer. In 3A, carbon revenue ranges from $0.72bn to $1.36bn (base $1.04bn) and deaths avoided from 36 to 68 (base 52); coverage moves with the block’s size, from 9.5 to 17.7% (base 13.5%). Higher block output raises the cut in every scenario, because CPAT’s own cut for the CBAM products is fixed while the block’s cut grows with its size.

As a plausibility check, the block’s baseline is 27.7 Mt of fuel CO2 and 31.0 Mt of process CO2 in 2030, which is 31% of CPAT’s industry energy CO2 (88.4 Mt) and 36% of its IPPU (85.5 Mt). Calibrating the levels and growth rates to CPAT’s baseline, and to CAPMAS and UN Comtrade data, would remove most of this sensitivity.

# Appendix F. The fuel-intensity response against the literature

F.1 What is checked. The block’s βf=0.0058 (Appendix B) means a $20 price cuts fuel CO2 per tonne about 11%. The literature gives it as energy-price elasticities with output fixed, converted here at Egyptian fuel prices, and as observed falls in emissions under real carbon prices.

F.2 Converting to a carbon price. At $20 per tonne of CO2, the charge is about $1.1 per MMBtu of gas, $1.9 per GJ of coal and $2.0 per GJ of petroleum coke. Egypt’s industrial gas prices are $7.75 per MMBtu for steel and $14 for cement from May 2026, with nitrogen fertilizer plants on formula prices of roughly $6 to $8. Imported coal and petroleum coke cost about $5 and $3.5 per GJ. The charge raises gas costs about 15%, coal 40% and petroleum coke 55%: about 30% on the products’ fuel bill.

F.3 The evidence.

| Source | Finding | Fall in fuel CO2 per tonne at $20 | βf |
| --- | --- | --- | --- |
| Labandeira et al. (2017), meta-analysis | Energy demand −0.65 long run (gas −0.61); industry less responsive, about −0.3 to −0.6 with output fixed | 8 to 14% (30% fuel cost rise) | 0.0037 to 0.0075 |
| Dechezleprêtre et al. (2023), EU ETS | Emissions down about 10% in 2005 to 2012, no loss of revenue or jobs; prices €20 or less | About 10% | About 0.005 |
| Colmer et al. (2025), EU ETS, French manufacturing | Emissions down 14 to 16% through lower intensity; no fall in output | 14 to 16% | Up to 0.008 |
| Döbbeling-Hildebrandt et al. (2024), 80 ex-post studies of 21 schemes | Emissions 5 to 21% lower (4 to 15% after bias correction), mostly at low prices | Economy-wide, includes activity | Not per dollar |
| Martin et al. (2014), UK Climate Change Levy | Levy of about 15% of energy prices cut energy intensity 17%, mostly electricity | Mostly Scope 2, outside CBAM | Upper bound |

The consensus is a fall of about 10% at $20, so βf≈0.005, with a range of 0.0037 to 0.0075. The block’s 0.0058 sits inside the range, about 15% above its center. CPAT’s own efficiency elasticity (about −0.45) gives 0.0056, almost the residual, which supports Appendix B. The ex-post studies cover European firms; Egypt’s newer fertilizer and steel plants may have less room to cut, and its coal- and coke-fired cement kilns more.

F.4 How the results change. The workbook has an override cell for βf (Calculations, row 86). With the consensus value of 0.005 (now → consensus):

| Scenario | Emission cut, Mt | Heavy-industry intensity | Border charge per tonne | Deaths avoided |
| --- | --- | --- | --- | --- |
| 1A | 39.8 → 39.4 | −8.7% → −8.0% | −38.7% → −38.2% | 1,617 → 1,594 |
| 2A | 36.8 → 36.4 | −5.2% → −4.5% | −20.7% → −20.2% | 1,597 → 1,574 |
| 2B | 40.4 → 40.0 | −5.2% → −4.5% | −20.7% → −20.2% | 1,757 → 1,733 |
| 3A | 6.9 → 6.5 | −8.7% → −8.0% | −38.7% → −38.2% | 52 → 46 |
| 3B | 5.1 → 4.7 | −8.7% → −8.0% | −8.5% → −7.7% | 38 → 33 |
| 3C | 11.5 → 10.8 | −16.6% → −15.3% | −43.9% → −42.9% | 86 → 75 |

Coverage and revenue do not change. Across the full range (0.0037 to 0.0075), the 3A cut runs from 5.9 to 7.7 Mt, block intensity in 1A from −6.9% to −10.1%, and the border charge in 1A from −37.4% to −39.7% and in 3B from −6.5% to −10.0%.

The choice matters most for 3A to 3C; it barely moves 1A to 2B or the credited part of the border charge. No conclusion in section 1 changes.

Sources. Colmer, Martin, Muûls and Wagner (2025), Review of Economic Studies; Dechezleprêtre, Nachtigall and Venmans (2023), Journal of Environmental Economics and Management 118; Döbbeling-Hildebrandt et al. (2024), Nature Communications 15; Labandeira, Labeaga and López-Otero (2017), Energy Policy 102; Martin, de Preux and Wagner (2014), Journal of Public Economics 117; Egypt gas prices: Official Gazette, effective May 1, 2026 (Business Today Egypt).

# Appendix G. The heavy-industry base: CPAT and the CBAM block

G.1 Why the base is smaller than in v1.0. In v1.0 (June), the downstream scenarios took CPAT’s own run as the base: energy CO2 of all industry plus process emissions of the CBAM sectors, about 20% of national GHG, and a cut of 21.5 Mt in 3A. This version prices only the seven CBAM products. Their baseline in 2030 is 58.8 Mt (27.7 Mt from fuel, 31.0 Mt from processes), 13.5% of national GHG, and the cut in 3A is 6.9 Mt.

G.2 The comparison (2030 baseline, Mt CO2e; CPAT’s industry fuel split uses IPCC emission factors on its energy use, scaled to its total).

| Emissions | CPAT (1A run) | CBAM block | Block share |
| --- | --- | --- | --- |
| Industry energy CO2 | 88.4 | 27.7 | 31% |
| Natural gas | 31.6 | 11.5 (DRI, ammonia, urea, cement) | 36% |
| Coal | 8.7 | 8.3 (cement kilns) | 96% |
| Other oil products (petroleum coke, fuel oil) | 14.4 | 7.9 (cement kilns) | 55% |
| Diesel | 23.3 | 0 | 0% |
| LPG | 10.4 | 0 | 0% |
| Iron and steel only, energy CO2 (1A policy values; CPAT reports no baseline) | 4.8 | 3.9 | 80% |
| Industrial processes (IPPU) | 85.5 | 31.0 | 36% |
| Total | 173.9 | 58.8 | 34% |

The products account for nearly all of CPAT’s industrial coal and, for iron and steel, most of its energy CO2. The difference lies elsewhere: diesel and LPG (34 Mt) are used by other industry, construction and mining, and most industrial gas burns in other manufacturing, such as ceramics, glass, food and textiles. On the process side, CPAT’s 85.5 Mt includes lime, glass, chemicals and fluorinated gases, which the EU CBAM does not cover; the ammonia feedstock CO2 (3.8 Mt) that inventories count as a process emission is counted here as fuel.

G.3 What CPAT cannot do. For Egypt, CPAT’s sub-industry energy data are empty except for iron and steel: cement, chemicals and non-ferrous metals show zero energy use, and in the full results export the sub-industry rows are misaligned with their codes (rows labeled “other manufacturing” carry the iron and steel codes). So a CPAT run cannot price the CBAM sectors alone. Its “downstream” run prices fuel used by all industry, including diesel and LPG outside the seven products, and, before the fix, transport fuels as well. v1.0 inherited this, which is why its base was about twice as large and its cut three times larger.

G.4 What the study now does. For 3A to 3C, the base is the CBAM block alone: coverage is the block’s emissions over national GHG (13.5%), and the cut, revenue and border charge all come from the block. CPAT is used only for what it does well here: the rest of the economy in 1A to 2B, the industry fuel response (Appendix B), and deaths avoided. The block’s base rests on output and emission factors that can be checked plant by plant (Appendices A and D), which CPAT’s aggregate industry cannot.

CBAM products (heavy industry in this table): the seven goods covered by the EU’s Carbon Border Adjustment Mechanism (CBAM) that Egypt makes (DRI-EAF and scrap-EAF steel, gray clinker, ammonia, urea, ammonium nitrate, primary aluminum). National emissions use the shares of Egypt’s 2022 inventory, where energy CO2 is about 65% of the total, against 53% in CPAT’s 2030 baseline. 3A to 3C price the seven products only, so their coverage is the products’ own emissions; CPAT cannot isolate them (Appendix G). CPAT’s 3A and 3C runs imply implausibly more deaths from fuel switching, so their deaths avoided are 3B’s scaled by the emission cut.

Public investment raises GDP (about 0.4% more in 2030 than household refunds, in CPAT), so 2A cuts less than 2B. Household refunds go to the bottom 40% of households, with 90% coverage.

Gross where revenue funds public investment or household refunds; net, and so zero, where it is rebated to firms (3B) or put into a fund (3C).

Emissions per tonne of output. Heavy industry: products weighted by their emissions at policy output. EU border charge: products weighted by their EU exports. This assumes the EU recognizes the lower intensities, for example because Egyptian exporters report verified actual emissions under the CBAM monitoring, reporting and verification (MRV) rules rather than EU default values.

(1 + total change) ÷ (1 + intensity change) − 1: the output change, weighted by each product’s emissions.

The EU deducts the carbon price effectively paid in Egypt: the carbon price weighted by its coverage of the exported products’ emissions. Under the current interpretation of the rules, an output-based rebate (3B) invalidates the credit; an abatement fund (3C) does not, but earns no extra credit.

Annual, not cumulative. In 1A to 2B, mostly from cleaner air as fuel use falls in food and agriculture (about 60% of the fall in fine-particle pollution in CPAT) and road transport (about 25%).