Egypt carbon pricing and CBAM: what changed from the original results

Timing caveat: the results apply long-run responses in full by 2030. With a carbon price rising linearly from zero in 2026 to USD20 in 2030, published adjustment speeds imply about 40% of the response by 2030 (about 70% if the full price applies from 2027), so 2030 emission cuts, intensity gains and deaths avoided would be smaller in proportion (methodology Appendix H).

Main changes. The results now use a dedicated CBAM module for the seven CBAM goods Egypt makes (steel, clinker, ammonia, urea, ammonium nitrate and primary aluminum). CPAT's treatment of these sectors was weak: its industry energy balance for Egypt is suspect, with no sub-industry data except iron and steel, so a CPAT "downstream" run priced the fuel of all industry, including diesel and LPG well outside the CBAM goods. The 3A to 3C scenarios now cover the CBAM goods only, as intended, which cuts their base from about 20% to 15% of national emissions. The CPAT runs for the rest of the economy were also updated with better Egypt-specific prices. Finally, the ad hoc adjustments to CPAT had become hard to follow, and some assumptions were unsourced, so the study now has a standalone Excel workbook and methodology in which every assumption is sourced.

Effect on the results. For the economy-wide scenarios (1A to 2B), the results are similar: revenue, emission cuts and deaths avoided move by about 10% or less. The downstream scenarios (3A to 3C) change most. Their emission cuts fall by about half to two-thirds and their health benefits by about 85%, because they now count only the CBAM goods, and are much more accurate for it. The split between lower emissions per tonne and lower output is also more accurate, as the module models each separately for each product. Reductions in CBAM obligations are larger, mainly because the credit for Egypt's carbon price is now measured against the EU price after the 2030 CBAM phase-in (about USD61 per tonne), so a USD20 Egyptian price offsets a larger share; free allocation (3B) still earns no credit.

Results in 2030: original → current

| Scenario | National emissions priced | Revenue, USD bn | Emission cut, Mt | CBAM-sector intensity | CBAM obligation per tonne | Deaths avoided a year |
| --- | --- | --- | --- | --- | --- | --- |
| 1A Comprehensive levy | 72% → 71.5% | 5.8 → 5.6 | 41.6 → 40.7 | −7.6% → −8.9% | −26.1% → −38.6% | 1,656 → 1,659 |
| 2A Upstream, public investment | 65% → 64% | 5.2 → 5.0 | 38.9 → 37.5 | −5.5% → −5.5% | −13.9% → −20.6% | 1,564 → 1,639 |
| 2B Upstream, household refunds | 65% → 64% | 5.2 → 5.0 | 41.0 → 41.1 | −5.5% → −5.5% | −13.9% → −20.6% | 1,631 → 1,799 |
| 3A Downstream, household refunds | 20% → 15% | 1.1 → 1.1 | 21.5 → 7.8 | −7.6% → −8.9% | −26.1% → −38.6% | 546 → 61 |
| 3B Downstream, free allocation | 20% → 15% | 0.0 → 0.0 | 18.1 → 5.8 | −7.6% → −8.9% | −7.6% → −8.4% | 345 → 45 |
| 3C Downstream, abatement rebate | 20% → 15% | 0.0 → 0.0 | 24.5 → 13.0 | −13.1% → −16.9% | −30.4% → −43.7% | 621 → 101 |

Notes: Shaded rows are the CBAM-only scenarios, where the change is largest. All scenarios use a carbon price of USD20 per tonne in 2030. National emissions are now measured on the sector shares of Egypt's 2022 inventory. In 3A to 3C, diesel and gasoline are excluded from the price. The CBAM obligation is the change per tonne exported after crediting Egypt's carbon price. The original table showed an 18.1 Mt cut for 3B where its text gave 13.6 Mt. Sources: original results table; current workbook and methodology (v2.9).